Unit Commitment

For 1:N Generators()

Costs represented in the objective function ($/MWh, start-up, shut-down, no-load)

Price variable constraints, bidding segment constraints

Minimum capacity, maximum capacities and other constant variables

Upper bounds, lower bounds, variable array (integer/continuous). Every

subcomponent for the unit commitment has the constraints represented in matrix

form for A, Ae, b, be, xint, lb,ub, and f.

Summary – When the function completes, each generator will have a MIP objective function, equality and inequality arrays, upper bounds, lower bounds, and x-array. Since the generator constants do not change for each hour, there will be only one MIP for each generator usable for hours 1:N. These will be master reference arrays for calculating other components of the program. Each subcomponent for the unit commitment is represented inside the generator function because the generator influences the unit commitment. Each subcomponent also generates its own f, A, Ae, b, be, xint, lb, ub, and f matrices. When the subcomponent procedure is complete, the results are added to the master matrices representing the unit commitment MIP.

Inputs:

(noload,startupcost,shutdowncost,price,quantity,NT,PMIN,PMAX,QSC,MSR,RU,RD,D,SR,OR)

Outputs:

(f,Aeq,beq,Aeinq,beinq,lb,ub,x)

# Program Flow Diagram

## INPUTS

Generator = [price, bidding segment, MW, minimum capacity, maximum capacity, no-load cost, start-up cost, shut-down cost, minimum up time, minimum down time, ramp-up rate, ramp-down rate, MSR, QSC, Initial Status, Initial Hour, Initial MW, system load, spinning reserve, operating reserve]

Line = [from, to, r, x, flow limit]

Load = [bus, percentage of system load]

## INPUTS

Generator = [price, bidding segment, MW, minimum capacity, maximum capacity, no-load cost, start-up cost, shut-down cost, minimum up time, minimum down time, ramp-up rate, ramp-down rate, MSR, QSC, Initial Status, Initial Hour, Initial MW, system load, spinning reserve, operating reserve]

Line = [from, to, r, x, flow limit]

Load = [bus, percentage of system load]

Security

For 1:N Buses()

Imports bus and lines data. Computes admittance matrix and calculates power transfer distribution factors. Limits on the line flows are entered as constraints along with phase shift limits.

Summary – The PTDF are calculated for N buses for K lines. The constraints hold PTDF information and flow limits

Inputs:

(line, frombus, tobus, r, x, flowlimit, pstrans, frombus, tobus, r, x , anglemax , anglemin, load, busno, percent)

Outputs:

(f,Aeq,beq,Aeinq,beinq,lb,ub,x)